Instruction: Encircle the correct answer.

BUILDING UTILITIES

- PLUMBING & SANITARY

- Ancient roman term which refers to an individual working in the sanitary field.

- Plumber

- Plumbarius

- Plumbarium

- Plumbarian

- Pipes that are used for city water.

- Cast iron pipes

- Steel pipes

- Galvanized iron pipes

- PVC pipes

- Pipes that are used for pipelines, truck mains, and inverted siphon where pressures are high and sizes are large.

- PVC pipes

- Steel pipes

- Galvanized iron pipes

- Cast iron pipes

- It is a stronger and more elastic type of cast iron used in modern plumbing installation.

- Gray cast iron

- Ductile iron

- Malleable iron

- White cast iron

- The birth of the plumbing profession in the Philippines was traced back to the ______ century.

- 19th century

- 18th century

- 17th century

- 20th century

- He became the first Chief of the Division of Plumbing Construction and Inspection.

- Jaime M. Cabase

- Engr. Fortunato Amosco

- Hermogenes Pobre

- John F. Hass

- An association organized by Francisco Geronimo et. Al. in 1935.

- National Waterworks and Sewerage Authority (NAWASA)

- National Master Plumbers Association of the Philippines (NAMPAP)

- Board of Master Plumbers (BOMP)

- None of the above.

- What is the recommended slope for house drain?

- 1%

- 2%

- 3%

- 5%

- The length along the center of the pipe and fitting.

- Diameter

- Radius

- Developed length

- None of the above

- Act regulating the practice of Sanitary Engineering in the Philippines.

- RA 1364

- RA 9003

- RA 1634

- RA 1378

- When was RA 1378 also known as Plumbing Code of the Philippines was approved?

- July 26, 1955

- June 26, 1955

- July 28, 1955

- June 28, 1955

- A pump that is used to raise water from shallow depths and used most frequently for individual houses.

- Low lift pump

- High lift pump

- Hand pump

- Booster pump

- A pump that discharges treated water into arterial mains.

- Low lift pump

- High lift pump

- Hand pump

- Booster pump

- A pump that lifts surface water and moves it to a nearby treatment plant.

- Low lift pump

- High lift pump

- Hand pump

- Booster pump

- A pump that increases the pressure within the distribution system to an elevated water tank.

- Low lift pump

- High lift pump

- Hand pump

- Booster pump

- True or False: Centrifugal casting on metal moulds is sometimes called de Lavaud process.

- True

- False

- What is the equalizing volume of a water tank that is about of the total daily demand?

- 1/6 to 1/3

- 1/3 to 1/4

- 1/3 to 1/2

- 1/6 to ¼

- Type of underground water found on islands or coastal regions near seas.

- Mineral water

- Fresh water

- Spring water

- Well water

- The following are the three (3) main sources of water supply except

- Rainwater

- Ground water

- Surface water

- Underground water

- The minimum distance of a well from a septic tank.

- 60 meters

- 45 meters

- 15 meters

- 25 meters

- What is the minimum size of a swimming pool drain?

- 2 ½ ” dia

- 2” dia

- 3” dia

- 4” dia

- The quality of galvanized iron pipes is rated by?

- Diameter

- Gauge

- Schedule

- Thickness

- A valve that prevents water hammer.

- Butterfly valve

- Float valve

- Pressure relief valve

- Globe valve

- A pipe fitting shaped like letter “S”.

- U-trap

- P-trap

- Gully trap

- Bell trap

- What is the minimum number of water closet required for female population of an elementary school?

- 1 per 25

- 1 per 15

- 1 per 35

- 1 per 45

- Black steel pipe with galvanized malleable fitting is ideal for?

- Chemical Pipe

- Medical waste pipe

- Gas pipe

- None of the above.

- A loud banging noise that occurs when a liquid in motion is forced to change direction or stop suddenly, causing a pressure surge or shockwave in a piping system.

- Backflow

- Water hammer

- Back siphonage

- Backpressure backflow

- Backflow due to negative pressure.

- Backflow

- Water hammer

- Back siphonage

- Backpressure backflow

- Occurs due to increase reverse pressure above the supply pressure.

- Backflow

- Water hammer

- Back siphonage

- Backpressure backflow

- What is the minimum width of septic tank?

- 0.90 mts

- 1.00 mts

- 1.20 mts

- 1.50 mts

- What is the minimum length of septic tank?

- 1.20 mts

- 1.50 mts

- 2.00 mts

- 2.50 mts

- What is the minimum liquid depth for a septic tank?

- 0.50 mts

- 0.70 mts

- 0.90 mts

- 1.00 mts

- What is the minimum capacity of the secondary compartment of a septic tank?

- 1.00 cum

- 1.20 cum

- 1.50 cum

- 1.80 cum

- What is the minimum dimension septic tank manhole?

- 606 mm

- 506 mm

- 608 mm

- 508 mm

- What is the minimum height of a water seal for each fixture trap?

- 50mm

- 51mm

- 52mm

- 54mm

- What is the maximum height of a water seal for each fixture trap?

- 100mm

- 102mm

- 104mm

- 108mm

- Type of water closet that is noisy but highly efficient where strong jet into up leg forces contents out.

- Siphon vortex

- Siphon jet

- Reverse trap

- Blowout

- Type of water closet that is sanitary, efficient, and very quiet where water enters through the rim and through the down leg.

- Siphon vortex

- Siphon jet

- Reverse trap

- Blowout

- Type of water closet that is least efficient, subject to clogging, noisy, and use a simple washout action through a small irregular passageway.

- Blowout

- Reverse trap

- Washdown

- Siphon jet

- Type of water closet similar to siphon jet except that it has smaller trap passageway and moderately noisy.

- Siphon vortex

- Reverse trap

- Blowout

- Washdown

- Color code for pipes containing acid.

- Black

- Green

- Red

- Yellow

- A vent which also serves as a drain.

- Unit vent

- Yoke Vent

- Stack Vent

- Wet vent

- A vent where one vent pipe will serve two traps.

- Unit vent

- Local vent

- Main vent

- Stack vent

- A vent which is an extension of a soil stack above the highest horizontal drain and its uppermost end above the roof.

- Yoke Vent

- Main Vent

- Stack Vent

- Local vent

- A vent installed to allow additional air circulation between drainage and vent systems.

- Wet vent

- Yoke vent

- Relief vent

- Stack vent

- A vent installed upward from a soil stack to a vent stack to prevent pressure changes.

- Wet vent

- Yoke vent

- Relief vent

- Stack vent

- Globe Valve

- Ball Valve

- Butterfly Valve

- Gate valve

- Check valve

- Globe Valve

- Ball Valve

- Butterfly Valve

- Gate valve

- Check valve

- Globe Valve

- Ball Valve

- Butterfly Valve

- Gate valve

- Check valve

- Globe Valve

- Ball Valve

- Butterfly Valve

- Gate valve

- Check valve

- Water traps are used to prevent foul odor from coming out. Which of the following does not have a water trap?

  - Lavatory

  - Kitchen Sink

  - Floor Drain

  - Water closet

- A valve right angle to the flow of water.

- Check Valve

- Globe Valve

- Gate Valve

- None of the above

- What is the use of chlorine?

- Water disinfectant

- Water purifier

- Conversion to mineral water

- All of the above

- It is added to water to reduce salinity

- 20 psi

- 40 psi

- 60 psi

- 80 psi

- Which pipe is susceptible to UV rays?

- Cast iron

- PVC

- GI pipe

- All of the above

- Copper wires are classified as what?

- Rigid

- Expensive

- Galvanized

- All of the above

- Which should not be used for water main pipe?

- Galvanized Iron

- Ceramic

- Stainless Steel

- PVC

- Where would you locate water distribution pipe farthest from?

- Large trees

- Carport

- Carpark

- None of the above

- Under a Concession Agreement that granted the company exclusive rights to the use of land and facilities for the production, treatment and distribution of water, as well as the rights to operate the sewerage system, which is now in-charge over the East Zone of Metro Manila?

a. Manila Water

b. MWSS

c. NAWASA

d. Maynilad

- 171. In 1997, this company was granted a 25-year exclusive concession to operate, maintain and invest in the water and sewerage system of the West Zone of Metro Manila. In 2010, the concession term was later extended by 15 years as approved by the Department of Finance.

a. Manila Water

b. MWSS

c. NAWASA

d. Maynilad

- You were hired to do some renovation works for a 6-storey apartment and you noticed that there is no Overhead Water Tank. What can you deduce about the structure?

a. It utilizes a Downfeed system b. It utilizes an Upfeed System

c. The Design is wrong

d. It utilizes Rain-Harvesting Technology

- What will be the recommended size in terms of gallons for a Water Tank that will serve the needs for a 2-Storey Residence with 7 occupants?

a. 300

b. 450

c. 500

d. 350

- You were tasked by a client to determine a suitable location for a subdivision project. Which would be your main consideration in determining the water source for the proposed community?

a. Location of Drainage

b. Location of Waste Sewer

c. Availability of Public Water

d. Depth of Wells

- Of the following fixtures inside a T&B, which has the greatest water pressure?

a. Faucet (lavatory)

b. Shower

c. Water Closet

d. All of the fixture has equal pressure

- From the figure above, identify A.

a. Curb Stop

b. Meter Stop

c. Corporation Stop

d. Gate Valve

- From the figure above, identify B.

a. Curb Stop

b. Meter Stop

c. Corporation Stop

d. Gate Valve

- From the figure above, identify C.

a. Curb Stop

b. Meter Stop

c. Corporation Stop

d. Gate Valve

- Copper pipes are classified as what?

a. Rigid

b. Rigid or Soft

c. Flexible

d. Soft

- Which should not be used for water main pipe?

a. Galvanized Iron

b. Stainless Steel

c. Ceramic

d. PVC

- Kind of pipe not suited for ultraviolet rays

a.	PB

b.	Iron

c.	PVC

- What pipe should not be buried underground?

a. BI Pipe

b. GI Pipe

c. Polyethylene Pipe

d. None ALL can be used underground

- Which type of piping material is NOT suitable for underground installation?

a. uPVC

b. Stainless Steel

c. Wrought Iron

d. Galvanized Iron

- As per Sanitary Engineering Code, what is the color code for Drainage and Sewage Pipe?

a. Orange

b. Black

c. Light Orange

d. Blue

- As per Building Code, what is the color code of pipes carrying saltwater

a. Green

b. Orange

c. Blue

d. Yellow

- Which is the most commonly used material for cold water line?

a. PPR

b. CPVC

c. GI

d. PVC

- What is the commercial length of a GI Pipe in the Philippines?

a. 3m

b. 5m

c. 4m

d. 6m

- What is the commercial length of a PVC Pipe in the Philippines?

a. 3m

b. 5m

c. 4m

d. 6m

- What is the commercial length of a PPR Pipe in the Philippines?

a. 3m

b. 5m

c. 4m

d. 6m

- Which is used for Sanitary Pipes?

a. PeX

b. GI

c. PP

d. uPVC

- An underground pit used for the temporary collection and storage of feces, excreta or fecal sludge as part of an on-site sanitation system. It is usually lined by bricks or concrete, covered with a slab and needs to be emptied frequently.

a. Privy

b. Septic Tank

c. Cesspool

d. Latrine

- If the site has no existing sewer system, what is your solution as per National Plumbing Code?

a. Provide Septic Tank

b. Use the river

c. Throw waste to the closest field

d. Provide Portalets

- What is the Minimum Water Level of a Septic Tank?

a. 1’6”

b. 2’6”

c. 2’

d. 4’

- A pit for the reception or detention of sewage/non-watertight lined excavation; permitting liquid to seep through the bottom and sides of the cesspool

a. Sump Pit

b. Cesspool

c. Septic Tank

d. Sewage Treatment Plant

- A water-tight receptacle; separate solids from the liquid, digest organic matter, allow effluent to discharge to a storm drain, directly to ground outside the tank through open joint or perforated piping

a. Sump Pit

b. Cesspool

c. Septic Tank

d. Sewage Treatment Plant

- Another term for Seepage Pit

a. Sump Pit

b. Cesspool

c. Septic Tank

d. Sewage Treatment Plant

- Consists of human excrement/liquid household waste

a. Wastewater

b. Sanitary Sewage

c. Waterborne Waste

d. Industrial Waste

- Pipe with or without industrial wastes/without and admixture of storm water or ground water

a. Sanitary Sewer

b. Sanitary Sewage

c. Domestic Pipe

d. Sanitary Pipe

- A vertical distance between the dip and the crown weir of a trap/the water in the trap between the dip and the crown weir

a. Water Seal

b. Trap Seal

c. Seal

d. Trap Arm

- The principal artery in vent system

a. Main Vent

b. VSTR

c. SVTR

d. Main Soil Vent

- The National Plumbing Code of the Philippines was approved by the Secretary of Public Works and Communications on what date?

A. January 28, 1959
B. October 28, 1959
C. December 21, 1959
D. February 20, 1959

- The minimum size of a water service pipe for any building is:

A. 12 mm
B. 19 mm
C. 25 mm
D. 38 mm

- A plumbing fixture that receives fecal matter and human excreta is called a:

A. Water closet
B. Urinal
C. Lavatory
D. Slop sink

- 4. The fitting used to connect two pipes of different sizes is a:

A. Coupling
B. Reducer
C. Union
D. Bushing

- The vertical vent pipe installed primarily for the purpose of providing air circulation to and from any part of the drainage system is called a:

A. Stack vent
B. Vent stack
C. Relief vent
D. Yoke vent

- What is the standard unit of pipe length in plumbing practice?

A. 1 meter
B. 2 meters
C. 3 meters
D. 6 meters

- A pipe that conveys the discharge of water closets, urinals, and similar fixtures is called:

A. Waste pipe
B. Soil pipe
C. Vent pipe
D. Drain pipe

- The minimum trap seal required in a plumbing fixture is:

A. 12 mm
B. 25 mm
C. 38 mm
D. 50 mm

- The main vertical pipe of a sanitary drainage system is called a:

A. Stack
B. Vent
C. Cleanout
D. Lateral

- What is the maximum spacing between cleanouts in a horizontal drainage line?

A. 6 m
B. 12 m
C. 15 m
D. 30 m

- The branch vent that connects two or more individual vents to a vent stack or stack vent is a:

A. Circuit vent
B. Common vent
C. Loop vent
D. Header vent

- The minimum slope for horizontal drainage pipes 102 mm (4 in) or smaller is:

A. 1%
B. 2%
C. 4%
D. 0.5%

- What is the minimum size of a vent pipe serving a water closet?

A. 25 mm
B. 32 mm
C. 38 mm
D. 50 mm

- The minimum number of water closets required for male users in a public toilet with 300 occupants is approximately:

A. 2
B. 3
C. 4
D. 5

- What is the maximum allowable distance of a fixture trap from its vent for a 38 mm (1½") diameter trap arm?

A. 0.9 m
B. 1.2 m
C. 1.5 m
D. 1.8 m

- In the design of a water supply system, the minimum residual pressure at the farthest fixture outlet on the top floor should not be less than:

A. 7 psi
B. 10 psi
C. 15 psi
D. 20 psi

- The minimum size of building drain serving a water closet is:

A. 50 mm
B. 75 mm
C. 100 mm
D. 150 mm

- The fixture unit value assigned to a standard water closet (tank type) is:

A. 2 FU
B. 3 FU
C. 4 FU
D. 6 FU

- The minimum slope for a 100 mm horizontal drainage pipe is:

A. 1%
B. 2%
C. 0.5%
D. 4%

- The air gap for indirect waste piping should not be less than:

A. Twice the diameter of the pipe
B. 1 inch
C. Three times the diameter of the pipe
D. 25 mm

- The minimum diameter of a vent stack shall not be less than:

A. One-half of the soil stack
B. Two-thirds of the soil stack
C. Equal to the soil stack
D. 32 mm

- The minimum number of stacks required in a three-story apartment building with 12 water closets is:

A. 1
B. 2
C. 3
D. Depends on layout and load

- A trap seal must be maintained between:

A. 25 mm to 50 mm
B. 38 mm to 76 mm
C. 19 mm to 51 mm
D. 50 mm to 75 mm

- In building drainage design, cleanouts shall be provided:

A. At every change of direction greater than 45°
B. Every 6 meters
C. Every 12 meters
D. Both A and C

- The minimum distance of a vent opening from any window, door, or air intake is:

A. 1.0 meter
B. 1.5 meters
C. 3.0 meters
D. 4.5 meters

- The fixture unit value for a lavatory is:

A. 1 FU
B. 2 FU
C. 3 FU
D. 4 FU

- The building sewer must begin at least how far from the building wall?

A. 0.3 m
B. 0.6 m
C. 1.0 m
D. 1.5 m

- The minimum diameter of a rainwater leader serving 100 m² roof area is approximately:

A. 50 mm
B. 75 mm
C. 100 mm
D. 125 mm

- The maximum number of 90° bends allowed between cleanouts in a horizontal drainage line is:

A. 1
B. 2
C. 3
D. None

- The minimum distance of the water closet centerline from a side wall is:

A. 150 mm
B. 200 mm
C. 300 mm
D. 450 mm

- A tool used to cut pipes to the desired length is called a:

A. Pipe reamer
B. Pipe wrench
C. Pipe cutter
D. Hacksaw

- The tool used to hold and turn pipes is the:

A. Monkey wrench
B. Pipe wrench
C. Chain wrench
D. Adjustable wrench

- Which tool is used to remove burrs and smooth the inside edge of a cut pipe?

A. Pipe threader
B. Pipe reamer
C. Die stock
D. File

- The tool used for cutting pipe threads is a:

A. Die stock and die
B. Pipe vise
C. Teflon tape
D. Reamer

- The tool used to hold pipes firmly while cutting or threading is called a:

A. Pipe vise
B. Pipe wrench
C. Chain wrench
D. Bench vise

- The chain-type wrench is used for:

A. Cutting large pipes
B. Gripping large diameter pipes
C. Measuring pipe length
D. Smoothing threads

- Which of the following tools is used to tighten or loosen nuts and bolts of different sizes?

A. Pipe wrench
B. Adjustable wrench
C. Chain wrench
D. Basin wrench

- A basin wrench is used specifically for:

A. Cutting small pipes
B. Loosening fittings under sinks
C. Threading pipes
D. Holding fittings on walls

- A plumber’s torch is used for:

A. Melting lead
B. Soldering or sweating copper pipes
C. Heating steel pipes
D. Cleaning pipes

- The tool used for bending pipes is a:

A. Pipe cutter
B. Pipe bender
C. Pipe vise
D. Pipe reamer

- A plumber’s auger or snake is used for:

A. Cleaning clogged drains
B. Threading pipe
C. Cutting pipes
D. Measuring slope

- The plumber’s level or spirit level is used to:

A. Check water pressure
B. Align pipes horizontally or vertically
C. Measure slope angle
D. Determine pipe size

- The plumber’s torch flame color that indicates the correct temperature for soldering copper pipes is:

A. Yellow
B. Red
C. Blue
D. White

- A lead pot and ladle are used in plumbing for:

A. Threading plastic pipes
B. Pouring lead joints in cast iron pipes
C. Cutting pipes
D. Bending copper pipes

- A plunger (force cup) is used to:

A. Remove pipe burrs
B. Unclog sinks and toilets
C. Tighten pipe fittings
D. Solder joints

- The minimum air gap between the water supply outlet and the flood level rim of a fixture shall be:

A. Equal to the pipe diameter
B. Twice the pipe diameter
C. Three times the pipe diameter
D. 25 mm

- A yoke vent is installed:

A. At the base of the stack
B. Between branch intervals
C. Above the highest fixture
D. Between fixture traps

- The minimum slope for a 50 mm horizontal drainage pipe is:

A. 1%
B. 2%
C. 4%
D. 0.5%

- The minimum distance of a water closet centerline from any side wall is:

A. 150 mm
B. 200 mm
C. 300 mm
D. 450 mm

- The minimum distance of a vent terminal from any window, door, or air intake is:

A. 1.0 m
B. 1.5 m
C. 3.0 m
D. 4.5 m

- The main stack or soil stack shall extend at least how high above the roof?

A. 150 mm
B. 300 mm
C. 450 mm
D. 600 mm

- What government agency regulates the licensing of plumbers in the Philippines?

a. Department of Health (DOH)
b. Department of Public Works and Highways (DPWH)
c. Professional Regulation Commission (PRC) 
d. Local Government Units (LGUs)

- What is the official plumbing standard followed in the Philippines?

a. Philippine Water Code
b. National Plumbing Code of the Philippines 
c. International Plumbing Standard
d. Bureau of Fire Protection Manual

- What type of pipe is most commonly used for water supply lines in residential areas in the Philippines?

a. Copper
b. Galvanized Iron
c. PVC (Polyvinyl Chloride) 
d. Lead

- What does a “P-trap” do in plumbing systems?

a. Increases water pressure
b. Stores extra water
c. Prevents sewer gases from entering the home 
d. Filters drinking water

- What is the minimum slope for a horizontal drainage pipe according to the plumbing code?

a. 1%
b. 2% (or 1/4 inch per foot) 
c. 5%
d. No slope required

- Which of the following is NOT a function of a vent pipe?

a. Allows sewer gases to escape
b. Equalizes pressure in drainage system
c. Carries waste water to septic tank 
d. Prevents siphoning of traps

- Which type of water closet uses the least amount of water per flush?

a. Traditional single flush
b. Pressure-assisted flush
c. Dual flush toilet 
d. Gravity flush toilet

- Who is authorized to sign plumbing plans for permit application in the Philippines?

a. Any engineer
b. Construction foreman
c. Licensed Master Plumber 
d. Barangay official

- What is the common cause of low water pressure in households?

a. High elevation
b. Leaking pipes
c. Clogged fixtures
d. All of the above

- What material is commonly used for sewer pipes in the Philippines?

a. Copper
b. PVC or HDPE 
c. Stainless steel
d. Aluminum

- In the Philippines, how often should septic tanks generally be cleaned or desludged?

a. Every year
b. Every 2 years
c. Every 3–5 years 
d. Only when full

- What fixture is required by the plumbing code in all bathrooms?

a. Bidet
b. Floor drain 
c. Bathtub
d. Urinal

- Which valve is used to stop water supply in emergencies?

a. Globe valve
b. Gate valve
c. Shut-off valve 
d. Check valve

- What does “backflow” mean in plumbing?

a. Flow of water under pressure
b. Drainage through gravity
c. Reverse flow of contaminated water into clean water supply 
d. Overflow of water tank

- What is the role of a “grease trap” in plumbing systems?

a. Trap large solids
b. Improve water pressure
c. Prevent fats, oils, and grease from entering drainage system ✔️
d. Filter drinking water

- What tool is commonly used to join PVC pipes?

a. Soldering iron
b. Solvent cement (PVC cement) 
c. Welding rod
d. Plumber’s tape

- Which fixture is used to control water flow at the point of use?

a. Main valve
b. Pressure regulator
c. Faucet (tap) 
d. Check valve

- What should you do first when fixing a leaking pipe?

a. Call the barangay
b. Apply sealant
c. Shut off the main water supply 
d. Open all faucets

- What type of plumbing pipe is most resistant to rust and corrosion?

a. Galvanized iron
b. PVC or PPR pipe 
c. Cast iron
d. Steel

- Which of the following best describes a "check valve"?

a. A valve used to increase pressure
b. A valve that filters water
c. A valve that allows water flow in one direction only 
d. A valve for hot water systems only

- What is the primary purpose of a septic tank?

a. Filter drinking water
b. Separate and decompose solid waste from wastewater 
c. Store rainwater
d. Increase water pressure

- When is a plumbing permit required in the Philippines?

a. Only for large buildings
b. Never required
c. For minor repairs only
d. For any new installation or major modification

- What test is commonly done to check for pipe leaks after installation?

a. Water discoloration test
b. Hydrostatic pressure test 
c. Smoke test
d. Electrical continuity test

- Which of the following contributes to water conservation in plumbing?

a. Leaving faucets slightly open
b. Using regular single-flush toilets
c. Installing aerators and low-flow fixtures 
d. Washing vehicles daily

- Which of the following is a potential health hazard if plumbing is poorly maintained?

a. Increased water pressure
b. Faster drainage
c. Contaminated drinking water 
d. Low electric bill

- What is the recommended material for hot water supply pipes?

a. PVC
b. CPVC (Chlorinated PVC) or PPR 
c. Galvanized iron
d. Rubber hose

- Which document must be approved before starting plumbing work in a new building?

a. Barangay clearance
b. Fire safety certificate
c. Plumbing plan with permit 
d. Delivery receipt

- What’s the danger of improper venting in plumbing systems?

a. Water flow becomes faster
b. Pipes expand
c. Sewer gases can enter the building 
d. More water is saved

- Which of these fixtures is not typically connected to a drainage system?

a. Lavatory
b. Water closet
c. Kitchen sink
d. Water meter

- According to the National Plumbing Code, who is responsible for the plumbing system inside a property?

a. Barangay
b. Water utility company
c. Property owner or occupant 
d. Fire department

- What material must plumbing fixtures be made of, per RNPCP Section on Materials – General Requirements?
A. Porous stone
B. Dense, durable, non-absorbent material with impervious surface
C. Uncoated cast iron
D. Untreated wood

- What is a “fixture branch” as defined in RNPCP?
A. The main building water supply line
B. The drain from two fixtures joined together
C. The piping between the fixture supply outlet and the distributing pipe
D. A vent stack

- Which trap type must be self-cleaning, except for interceptor traps or similar devices?
A. Every trap
B. Only floor drains
C. Only urinal traps
D. Only drum traps

- What is the support spacing requirement for screwed (threaded) pipe ≤ 19 mm diameter for horizontal piping?
A. ~1.5 m intervals
B. ~3.0 m intervals
C. ~4.5 m intervals
D. ~6.0 m intervals

- For copper tubing ≤ 38 mm diameter, what is the maximum spacing of hangers/backing supports?
A. 1.0 m
B. 1.8 m
C. 2.5 m
D. 3.6 m

- What is the minimum support interval for copper tubing ≥ 51 mm diameter?
A. ~1.8 m
B. ~2.4 m
C. ~3.0 m
D. ~3.6 m

- What is the definition of “flood level rim” in the RNPCP definitions?

A. Top edge of water heater
B. Level above which water begins to overflow a fixture
C. The inlet height in a tank
D. The backflow prevention device height

- According to RNPCP, what is a “plumbing unit”?

A. One water meter, one water closet, one lavatory, one shower head, one kitchen sink, one laundry tray, three floor drains, and four faucets/hose bibbs
B. One water closet + one lavatory only
C. All fixtures in building divided by number of plumbing zones
D. Any grouping of fixtures in a single bathroom

- What constitutes “private use” classification of plumbing fixtures per RNPCP?

A. Fixtures used only before 6 PM
B. Fixtures in residences, private bathrooms in hotels/hospitals, restricted rest rooms, etc.
C. Fixtures in public restrooms
D. None; all are public unless signified otherwise

- What is the term “residual pressure”?

A. Pressure with no flow
B. Pressure loss due to friction
C. The pressure at a fixture after all pressure drops from supply, piping, etc., under maximum demand
D. Pressure at the main supply only

- What is the requirement for overflow outlets when a fixture is provided with an overflow?

A. No overflow needed for kitchen sinks
B. Overflow must remain filled when stopper is closed
C. Waste to be arranged so standing water in the fixture cannot rise inside overflow when stopper is closed, and overflow remains empty when fixture is empty
D. Overflow outlet may discharge into trap freely

- What is not allowed as a method of connection in drainage or vent piping?

A. Using cleanouts
B. Drilling and tapping cast iron soil pipes for connections
C. Using standard fittings approved by Administrative Authority
D. Using change of direction fittings per code

- What is required of water closet bowls for public use, per RNPCP?

A. Round bowls only
B. Elongated bowl type with open front seats
C. Any bowl shape, seat optional
D. Floor mounted flush tanks only

- What gauge thickness is required for sheet metal sinks in restaurant kitchens (minimum)?

A. Gauge #20
B. Gauge #16 U.S. B&S (1.6 mm)
C. Gauge #12
D. Gauge #10

- In plumbing system repairs / alterations, deviations from code are permitted if:

A. They save cost
B. They are aesthetically pleasing
C. Approved by the Administrative Authority and necessary
D. Done only by registered plumbers

- What is the classification of an installation where bathroom is inside a family dwelling unit?

A. Public use
B. Private use
C. Semi-private use
D. Commercial use

- What is prohibited regarding the use of soil or waste pipes and vent pipes?

A. Using them in separate stacks
B. Run vent pipes below grade
C. Using vent piping as soil or waste pipe, or vice versa
D. Using same material for both

- What is “fixture drain” as defined in RNPCP?

A. Drain from fixture trap to the junction with other drains
B. Entire building sewer line
C. Any vent opening
D. Backwater valve connection

- What is “fixture supply” per RNPCP definitions?
A. Any water main in street
B. Water supply pipe connecting distributing pipe to fixture supply outlet
C. The water meter itself
D. Any faucet in the building

- What is required of in-ground piping when buried in the ground?
A. It can be placed on any soil type
B. No bedding needed
C. Laid on a firm bed for its entire length, or using a concrete cradle approved by Authority
D. It must be above ground only

- How should change of direction in drainage or waste piping be made?
A. With short bends or swept elbows only
B. With multiple sharp-angle fittings
C. Without fittings, by bending pipe
D. Change of direction is not allowed

- What is the requirement for vertical piping support in a multi-storey building?
A. Every 2nd floor only
B. At every one (1) meter interval
C. No supports needed for vertical pipes
D. Only at top and bottom of stack

- How far apart can supports (metal hangers) be spaced for small-size tubing (≤ 38 mm) horizontally without backing?
A. ~1 m
B. ~1.5 m
C. Larger spacing but with permitted supports; backing required for smaller sizes
D. ~2 m

- What defines “public use” in the classification of plumbing fixtures?
A. Use by public or non-family occupants
B. Use only during business hours
C. Use only if fee is charged
D. Use only by people outside the building

- What is required of water heater, storage device design and installation?
A. No insulation needed
B. Prevent danger from overheating or explosion
C. Must be located outside only
D. Use any available fuel type without restrictions

- What is the slope requirement for in-ground sewer/drainage pipe bedding?
A. No slope needed
B. Minimum slope as per drainage table in code (depends on pipe size)
C. Flat slope only
D. Slope less than 0.1% is acceptable

- How are sewage systems designed when there is no public sewer available?
A. No requirement cost-sharing
B. Must use private sewage-disposal system (e.g. septic tank) meeting RNPCP specs
C. Discharge into nearest stream
D. Use simple pit only

- What is the minimum number of water closets and kitchen-type sinks for each family dwelling unit abutting or having private sewage system?
A. One water closet and one kitchen sink; lavatory & bathtub/shower recommended
B. Two water closets and two kitchen sinks
C. One WC only
D. Kitchen sink only

- What is the requirement for backflow or back-siphonage protection in water supply systems?
A. Not addressed by RNPCP
B. Must prevent hazards of backflow or back-siphonage in pure water supply systems
C. Only required for industrial uses
D. Only for hot water systems

- What is “interceptor (clarifier)” as per RNPCP?
A. A trap for single fixture
B. Device to separate and retain deleterious or hazardous matters from wastes
C. Water meter in supply line
D. Vent opening

- What is required of joints and exposed tubing traps less than a certain gauge in metal traps?
A. No marking needed
B. Must have manufacturer’s name stamped, and gauge if tubing trap
C. Can be any sold by local hardware
D. Must be welded permanently

- What is Table 7-3 about in RNPCP?
A. Fixture supply units vs water heater rating
B. Discharge capacity (liters per second) per fixture unit for intermittent flow
C. Material strength of pipes
D. Minimum number of fixtures by occupancy type

- How many Fixture Units (FU) should be allowed for clothes washers in groups of three or more?
A. Counted individually per manufacturer rating
B. Rated at six FU each for common horizontal & vertical waste pipe runs
C. No aggregate FU needed
D. Half the rating of a single unit

- According to RNPCP, what must be furnished in plumbing works of a public building under the Technical Specifications?
A. Only cold water lines
B. Hot water lines optional
C. Both potable water line, sewer line, sanitary system, storm drain system, fixtures, piping, fittings, equipment etc., complete design included
D. Fixtures only, design assumed known

- What is the minimum slope allowed for pipes (storm, sanitary, etc.) in design/build contracts per local technical specifications noted in documents?
A. 0.5%
B. 1%
C. 1.5%
D. 2%

- What is required of cleanouts and access for concealed or ceiling-installed valves?
A. No access required if small enough
B. Profile drawings only
C. Valves concealed or in ceiling must have access manhole
D. Only show valves on plan, no physical access needed

- How must horizontal piping larger than small tubing be supported when it does not require backing?

A. Not supported at all
B. With spaced metal hangers
C. With wooden blocks only
D. With ropes or chains

- What is required for exhaust in septic or private sewage systems?
A. No venting needed outdoors
B. Trapped vent only indoors
C. Must vent to outer air, via house sewer system or other approved venting
D. Use mechanical fans only

- What is the standard regarding roof gutters’ drains in certain bid documents?
A. No strainers required
B. Strainers required on gutter drains
C. Gutter drains must discharge to sanitary line
D. Gutter drains optional for small roofs

- In plumbing materials’ specification, what standard applies for fixtures, fittings, accessories (as per certain procurement documents)?
A. Must be cheapest price only
B. Must be certified by BPS (Bureau of Philippine Standards)
C. No certification needed if installed properly
D. International standards only

- What is required of potable water piping and drainpipes color-coding per RNPCP or procurement documents?
A. No color coding required
B. Potable water pipes: blue; Drainpipes: gray, orange or brown etc.
C. All piping painted the same color
D. Color coding only applies in hospitals

- What is the requirement for the minimum thickness of plumbing fixture/water storage tank walls in septic design quoted in some bid documents?
A. No minimum thickness
B. 100 mm only
C. 200 mm reinforced concrete wall with waterproofing for septic tank walls
D. 50 mm brick wall

- What is the requirement for the septic tank cover and outlet pipes relative to finished floor line in some RNPCP spec interpretations?
A. At same level as finished floor
B. Elevated minimum of 1 foot above finish floor line
C. Below finish floor line for gravity flow
D. No specification

- What is required for fixture material surfaces?
A. Rough, non-glazed ceramics only
B. Smooth, impervious surfaces, free from unnecessary concealed fouling surfaces
C. Can be unfinished if painted
D. Surface texture doesn’t matter

- What must all horizontal branches or waste lines have when serving appliances with continuous flow?
A. Smaller diameter to restrict flow
B. Capacity sized to handle peak load as per Table 7-3 discharge capacities
C. No vent required
D. Only one fixture per branch

- How are intermittent flows vs continuous flows treated in discharge capacity tables?
A. Same treatment
B. Continuous flows allow more fixture units per L/s in table
C. Continuous flows are ignored
D. Intermittent flows always require larger pipe diameter

- What is the requirement for replacing or repairing plumbing in existing buildings?
A. Must meet full current code unless relief is granted by authority
B. Can ignore code if old building
C. Only visible fixtures must comply
D. Owner may waive compliance

- What is source of potable water supply demand term in code?
A. Domestic demand only
B. Maximum demand including combined hot & cold where applicable, simultaneous usage etc.
C. Only cold water considered
D. Only peak hour usage

- What is allowed in alternative special-use fixtures for materials?
A. Only porcelain enamel
B. Soapstone, chemical stoneware, leaded copper base, steel alloys if suit-ably lined or resistant
C. Uncoated metals only
D. No special materials allowed

- What requirements does RNPCP place on exposed plumbing fixture traps?
A. No marking required
B. Exposed traps less than certain gauge must be drawn brass etc., with manufacturer’s name, gauge, etc.
C. All traps may be PVC
D. Traps may be undersized if aesthetic demands dictate

- What is the minimum diameter of a building sewer according to the Revised National Plumbing Code of the Philippines?
A. 50 mm
B. 75 mm
C. 100 mm
D. 150 mm

- What is the required minimum slope for a 100 mm horizontal drainage pipe?
A. 0.5% (1:200)
B. 1% (1:100)
C. 2% (1:50)
D. 10% (1:10)

- What type of pipe material is NOT allowed for hot water supply piping?
A. CPVC
B. Copper
C. Galvanized Iron
D. PVC

- According to the Plumbing Code, how far should a cleanout be located from the building?
A. 0.30 meters
B. 0.60 meters
C. 1.20 meters
D. 3.00 meters

- The fixture unit value of a water closet (flush tank type) is:
A. 2
B. 3
C. 4
D. 6

- What is the minimum number of plumbing fixtures required for a commercial building with 25 male and 25 female occupants (office-type)?
A. 1 male WC, 1 female WC, 1 lavatory each
B. 2 male WC, 2 female WC, 2 lavatories each
C. 2 male WC, 1 urinal, 2 female WC, 2 lavatories each
D. 3 male WC, 3 female WC, 3 lavatories each

- Which of the following traps is NOT approved for use in plumbing systems?
A. P-Trap
B. S-Trap
C. Bottle Trap
D. Drum Trap

- A vent pipe that connects several individual vents to a vent stack is called:
A. Loop vent
B. Circuit vent
C. Relief vent
D. Branch vent

- What is the minimum vertical clearance between a water supply outlet and a flood-level rim of a fixture (air gap)?
A. 12 mm
B. 25 mm
C. Twice the diameter of the pipe
D. 32 mm

- A building with multiple floors and similar bathroom layout may use what kind of waste and vent system?
A. Single-stack system
B. Two-pipe system
C. Overhead drainage
D. Combination system

- What is the minimum horizontal slope for drainage piping sized 51 mm?
A. 0.5% (1:200)
B. 1% (1:100)
C. 2% (1:50)
D. 5% (1:20)

- According to Table 7-2, what is the drainage fixture unit (DFU) value for a public water closet (flush tank)?
A. 4 DFU
B. 5 DFU
C. 6 DFU
D. 3 DFU

- What is the minimum trap diameter (or trap & trap arm size) required for a bathtub or bidet?
A. 25 mm
B. 32 mm
C. 38 mm
D. 51 mm

- Which of the following is the correct DFU for a laundry tub?
A. 1
B. 2
C. 3
D. 4

- What is the DFU value assigned to an interceptor for sand, auto-wash, etc.?
A. 3
B. 4
C. 6
D. 8

- In the minimum plumbing facilities table (Table 4-1), for an office building, how many lavatories per person (male) are required?
A. 1 per 100 persons
B. 1 per 75 persons
C. 1 per 50 persons
D. 1 per 200 persons

- For showers in a public setting (each head), what is the water supply fixture unit (WSFU) assigned?
A. 2 WSFU
B. 3 WSFU
C. 4 WSFU
D. 5 WSFU