Instruction: Encircle the correct answer.

BUILDING UTILITIES

- PLUMBING & SANITARY

- This type of fixture can be served by a combination waste and vent system.

- Bidets

- Water closet

- Urinal

- Sinks

- If you increase vents by one pipe size, what will happen?

- It will allow vents of longer lengths

- It will allow vents to be shorter

- It will allow a uniform installation

- It will provide increased drainage

- A reservoir, tank or vessel for storing or holding water or other liquids.

- Water tank

- Water dam

- Coferdam

- Cistern/Cistern Tank

- Plumbing pipes with an Aqua-Blue band or stripe color coding are recognized as

- Air pipe

- Raw water pipe

- Gas pipe

- Potable water pipe

- The following are sources of water for domestic use except

- Sea water

- Rain water

- Surface water

- Ground water

- The process in which silt and dirt is removed from water.

- Screening

- Siltation

- Filtration

- Sedimentation

- The following are 3 stages of process in a water cycle. Which is not included?

- Precipitation

- Hydration

- Condensation

- Evaporation

- This fixture has the highest fixture unit rating.

- Water closet

- Kitchen sink

- Lavatory

- Bathtub

- Which of these is NOT a correct material permitted for drainage piping per the Philippine Plumbing Code summary?
A. ABS
B. Vitrified clay
C. Galvanized iron
D. Lead

- What is the minimum air gap required (when not affected by walls) for lavatories and other fixtures whose effective opening is ≤ 13 mm?
A. 13 mm
B. 25 mm
C. 38 mm
D. 51 mm

- Which trap type is disallowed in plumbing installations?
A. P-trap
B. S-trap
C. Drum trap
D. Bottle trap

- What is the maximum vertical length allowed for a 51 mm diameter vertical drainage pipe before it must receive branching (per Table 7-5)?
A. 14 m
B. 18 m
C. 20 m
D. 26 m

- What is the maximum DFU load allowed on a 102 mm (4 in) vertical drainage pipe?
A. 323 DFU
B. 484 DFU
C. 600 DFU
D. 1380 DFU

- Which chapter of the Revised National Plumbing Code deals with “General Regulations,” such as change of direction and grade?
A. Chapter 1
B. Chapter 2
C. Chapter 3
D. Chapter 4

- What is the required vertical distance (clearance) between inlet/outlet baffle in a septic tank above the water surface?
A. 50.8 mm
B. 101.6 mm
C. 152.4 mm
D. 304.8 mm

- In septic tank walls, the invert of the inlet must be at least how much higher than the invert of the outlet?
A. 25.4 mm
B. 38.1 mm
C. 50.8 mm
D. 76.2 mm

- Which fixture has the smallest DFU among these choices?
A. Dental unit / cuspidor
B. Floor drain
C. Clotheswasher
D. Bathtub

- What is the DFU for a floor drain?
A. 1
B. 2
C. 3
D. 4

- What is the maximum fixture unit loading for 38 mm horizontal drainage piping runs (per Table 7-5)?
A. 24 DFU
B. 48 DFU
C. 84 DFU
D. 143 DFU

- For a public water closet, what is its WSFU (water supply fixture unit) in the Philippine code?
A. 3 WSFU
B. 4 WSFU
C. 5 WSFU
D. 6 WSFU

- When are plumbing permits required under the Plumbing Code?
A. Only for new buildings
B. Only for major renovations
C. For new, alterations, repairs, or replacement of plumbing works
D. Only for commercial buildings

- hat is the term for the minimum standard quantity of plumbing fixtures forming a “unit” for design?
A. Plumbing load
B. Plumbing unit
C. Fixture module
D. Basic set

- What is “residual pressure” in plumbing terms according to the code?
A. Pressure when no flow exists
B. Pressure at source before losses
C. Pressure at a fixture after accounting for all losses
D. Pressure lost due to friction

- Which of the following is an example of an “indirect waste receptor”?
A. Floor drain directly connected
B. Receptor for coffee urn drained through air gap
C. Urinal trap
D. Cleanout

- What is the maximum distance a standpipe receptor for a clothes washer should extend beyond its trap as per code summary?
A. 4 m
B. 6 m
C. 8 m
D. 10 m

- Which plumbing fixture is assigned the highest DFU in Table 7-2?
A. Public water closet
B. Interceptor for sand auto wash
C. Clinic sink (flushing rim)
D. Urinal blow-out integral trap

- What is the DFU for a wash basin (lavatory) single?
A. 1
B. 2
C. 3
D. 4

- In septic tank design, which must be ventilated through the house sewer and to outer air?
A. Only septic tank cover
B. Inlet baffle only
C. Outlet baffle only
D. Inlet, outlet, compartments, vents equal to cross-sectional area

- Which pipe size allows a maximum vertical DFU of 84 (per Table 7-5)?
A. 38 mm
B. 51 mm
C. 65 mm
D. 76 mm

- Which of the following is true of drainage fittings in the plumbing code summary?
A. Interior surface may be rough
B. Fittings must reduce interior cross section
C. Fittings should be smooth interior of same diameter
D. Fittings can be smaller than main pipe

- In the minimum plumbing facilities table, for male in “assembly places” how many lavatories per person for the first 15 persons?
A. 1 per 15 persons
B. 1 per 10 persons
C. 1 per 40 persons
D. 1 per 100 persons

- Which plumbing fixture is primarily used for the disposal of human waste?
A. Sink
B. Bathtub
C. Toilet
D. Bidet

- True or False (choose the correct option):
A toilet plays a critical role in maintaining sanitation and hygiene.
A. True
B. False

- A toilet consists of a ceramic _____ connected to a plumbing system.
A. Seat
B. Pipe
C. Bowl
D. Lid

- Which type of toilet uses the force of gravity to flush waste?
A. Gravity-fed toilet
B. Pressure-assisted toilet
C. Vacuum toilet
D. Air-driven toilet

- Toilets are typically found in:
A. Garages
B. Basements only
C. Homes and commercial buildings
D. Offices only

- True or False (choose the correct option):
Pressure-assisted toilets use pressurized air to flush waste.
A. True
B. False

- Which of the following adds comfort and cleanliness to a toilet?
A. Handle
B. Seat and lid
C. Pedestal
D. Cover

- Toilets can also play a role in ______ design.
A. Engineering
B. Plumbing
C. Interior
D. Industrial

- Which plumbing fixture is used for washing hands and cleaning dishes?
A. Bathtub
B. Sink
C. Toilet
D. Faucet

- 10. True or False (choose the correct option):
Sinks are found only in bathrooms.
A. True
B. False

- The Sovent System is also known as what type of system?

- A. Two-Stack

- B. Vacuum

- C. Single-Stack

- D. Gravity

- Which of the following is NOT a component of the Sovent System?

- A. Building Drain

- B. Vacuum Pump

- C. Horizontal Branches

- D. Vertical Stacks

- What component is common in all three systems?

- A. Building Drain

- B. Vacuum Pump

- C. Vent Pipe

- D. Overflow Pipe

- The Sovent System uses how many stacks?

- A.One

- B. Two

- C. Three

- D. Four

- Which fitting in the Sovent System helps in separating air and water?

- A. Dearator Fitting

- B. Trap

- C. Vent

- D. Clean-out

- Which system operates by creating a pressure difference?

- A. Sovent System

- B. Vacuum System

- C. Gravity System

- D. Dual Stack System

- The Vacuum System requires what special equipment?

- A. Vacuum Pumps and Tanks

- B. Traps and Vents

- C. Clean-outs

- D. Gravity Drain

- Which component directs wastewater from the building to the public sewer?

- A. Horizontal Branch

- B. Building Sewer

- C. Building Drain

- D. Vertical Stack

- Which type of sanitary system uses both a waste stack and a vent stack?

- A. Sovent System

- B. Vacuum System

- C. Conventional Gravity System (Two-Stack)

- D. Dual Pressure System

- What is one advantage of using the Sovent System?

- A. Requires multiple stacks

- B. Saves space and materials

- C. Needs many vents

- D. Uses complex pumps

- What type of system uses vacuum devices for plumbing fixtures?

- A. Sovent

- B. Gravity

- C. Vacuum

- D. Pressure

- What component is found only in the Vacuum System?

- A. Building Sewer

- B. Vertical Stack

- C. Vacuum Devices

- D. Horizontal Branch

- Which system is commonly called a Two-Stack system?

- A. Conventional Gravity System

- B. Sovent

- C. Vacuum

- D. Double-Pump

- The Sovent System includes all the following EXCEPT:

- A. Horizontal Branch

- B. Vacuum Pump

- C. Dearator Fitting

- D. Vertical Stack

- Which component connects horizontal branches to vertical stacks?

- A. Building Drain

- B. Vent Pipe

- C. Fixture Branch

- D. Trap

- The principle of sanitary design states that pipes should:

- A. Be as long as possible

- B. Avoid clean-outs

- C. Take the shortest possible route

- D. Have multiple traps

- Control components like traps and vents should be placed:

- A. Randomly

- B. Strategically for efficiency

- C. Only at the sewer outlet

- D. Only at fixtures

- Which of the following is part of the Conventional Gravity System?

- A. Building Sewer

- B. Vacuum Device

- C. Dearator Fitting

- D. Vacuum Tank

- The Vacuum System operates based on:

- A. Gravity

- B. Negative Pressure

- C. Positive Flow

- D. Air Compression

- The Sovent System is a two-stack system.

- A. False

- B. True

- The Vacuum System requires vacuum pumps and tanks to operate.

- A. True

- B. False

- The Conventional Gravity System uses only one stack.

- A. True

- B. False

- Dearator fittings are used in the Vacuum System.

- A. False

- B. True

- Building Drain is common to all systems.

- A. True

- B. False

- The Sovent System eliminates the need for traps.

- A. True

- B. False

- Clean-outs should be located strategically for maintenance.

- A. True

- B. False

- The Vacuum System uses air pressure to move wastewater.

- A. False

- B. True

- Vertical stacks are not necessary in the Gravity System.

- A. False

- B. True

- The Sovent System requires multiple vent pipes.

- A. False

- B. True

- The Gravity System relies on pumps to move wastewater.

- A. True

- B. False

- A Vacuum System can operate without vacuum devices.

- A. True

- B. False

- The Building Sewer carries wastewater outside the building.

- A. True

- B. False

- Traps are part of sanitary system control components.

- A. True

- B. False

- Sovent Systems require vacuum tanks.

- A. True

- B. False

- The principle of design emphasizes efficiency.

- A. False

- B. True

- The Vacuum System uses dearator fittings.

- A. False

- B. True

- The Conventional Gravity System includes building drains and sewers.

- A. False

- B. True

- Sovent Systems reduce the need for multiple stacks.

- A. False

- B. True

- Control components are unnecessary in modern systems.

- A. True

- B. False

- The ________ system uses only one stack for waste and venting.

- A. Vacuum

- B. Sovent

- C. Gravity

- D. Dual Stack

- The ________ system operates by creating suction through pumps.

- A. Gravity

- B. Vacuum

- C. Sovent

- D. Single Stack

- The ________ system is also called a two-stack system.

- A. Sovent

- B. Vacuum

- C. Conventional Gravity

- D. Pressure

- In a sanitary system, the ________ carries wastewater to the public sewer.

- A. Building Sewer

- B. Vertical Stack

- C. Horizontal Branch

- D. Trap

- The Sovent System uses ________ fittings to separate air and water.

- A. Trap

- B. Vent

- C. Dearator

- D. Overflow

- The ________ should take the shortest possible route to the sewer.

- A. Pipes

- B. Traps

- C. Pumps

- D. Tanks

- ________ components like clean-outs and vents improve system efficiency.

- A. Drainage

- B. Control

- C. Auxiliary

- D. Vacuum

- The Vacuum System requires ________ for its operation.

- A. Vacuum Pumps and Tanks

- B. Multiple Stacks

- C. Gravity Drains

- D. Overflow Valves

- ________ branches connect fixtures to the vertical stacks.

- A. Horizontal

- B. Parallel

- C. Cross

- D. Vertical

- The ________ stack carries waste vertically in all systems.

- A. Vacuum

- B. Drain

- C. Vertical

- D. Pressure

- The Sovent System reduces materials because it has only ________ stack.

- A. Two

- B. Three

- C. One

- D. Four

- ________ devices are used in vacuum-operated plumbing fixtures.

- A. Vacuum

- B. Gravity

- C. Drain

- D. Vent

- The ________ system depends entirely on gravitational force.

- A. Sovent

- B. Vacuum

- C. Conventional Gravity

- D. Dual

- A building’s wastewater first passes through the ________ before reaching the sewer.

- A. Horizontal Branch

- B. Building Drain

- C. Vertical Stack

- D. Trap

- The ________ fittings are unique to the Sovent System.

- A. Trap

- B. Dearator

- C. Vent

- D. Pump

- ________ pumps help move wastewater in the Vacuum System.

- A. Air

- B. Vacuum

- C. Pressure

- D. Drain

- ________ control components improve sanitary system function.

- A. Vacuum

- B. Drain

- C. Strategically placed

- D. Randomly placed

- The two-stack system is also called the ________ gravity system.

- A. Dual

- B. Sovent

- C. Conventional

- D. Vacuum

- The principle of sanitary design emphasizes ________.

- A. Length

- B. Efficiency

- C. Decoration

- D. Pressure

- In the Sovent System, air and water separation occurs in the ________.

- A. Dearator Fittings

- B. Vacuum Pump

- C. Trap

- D. Vent

- The exterior piping that conveys sewage from the building drain to the point of disposal is called:

- A. Building Drain

- B. Waste Stack

- C. Building Sewer

- D. Vent Stack

- The lowest horizontal portion of the drainage system is known as the:

- A. Building Drain

- B. Building Sewer

- C. Vent Stack

- D. Soil Pipe

- The vertical extension of a building drain that receives discharge from horizontal branches is the:

- A. Vent Stack

- B. Stack Vent

- C. Waste/Soil Stack

- D. Building Sewer

- The vent for the stack that terminates to open air through the roof is the:

- A. Vent Stack

- B. Stack Vent

- C. Waste Stack

- D. Soil Pipe

- The main vent for the DWV system that receives other vents is called the:

- A. Stack Vent

- B. Waste Stack

- C. Vent Stack

- D. Soil Stack

- Which pipe conveys only wastewater without any fecal matter?

- A. Soil Pipe

- B. Waste Pipe

- C. Vent Pipe

- D. Branch Pipe

- Which pipe conveys discharge that includes fecal matter?

- A. Waste Pipe

- B. Soil Pipe

- C. Vent Pipe

- D. Stack

- The pipe used for ensuring air circulation in the plumbing system is the:

- A. Vent Pipe

- B. Waste Pipe

- C. Drain Pipe

- D. Stack Pipe

- The vertical main of a system extending through one or more stories is the:

- A. Branch

- B. Waste Pipe

- C. Stack

- D. Vent Pipe

- Any part of the piping system other than a main, riser, or stack is a:

- A. Branch

- B. Vent

- C. Sewer

- D. Drain

- The part of the plumbing system that receives discharge from the soil, waste, and other drainage pipes inside the building is the:

- A. Vent Stack

- B. House/Building Drain

- C. Stack

- D. Waste Pipe

- The extension from the house drain to the public sewer is the:

- A. Vent Stack

- B. Waste Pipe

- C. House/Building Sewer

- D. Branch

- No building sewer shall be less than:

- A. 50 mm

- B. 100 mm

- C. 150 mm

- D. 200 mm

- The soil pipe shall be properly:

- A. Painted

- B. Concealed or embedded in walls

- C. Exposed

- D. Covered with insulation

- The soil branch should be as:

- A. Short and direct as practicable

- B. Long as possible

- C. Curved

- D. Complex

- Soil pipe joints must be:

- A. Loose

- B. Tight and leak-free

- C. Open for inspection

- D. Flexible

- Soil pipes not embedded in concrete should be anchored using:

- A. Wooden braces

- B. Plastic ties

- C. Metal hangers

- D. Ropes

- Changes in direction from horizontal to horizontal should use:

- A. 45° wye branches or approved fittings

- B. 90° elbows

- C. Straight pipes

- D. Closed joints

- No fitting having more than one inlet at the same level shall be used; this type of fitting is called:

- A. Sanitary Cross

- B. Sanitary Tee

- C. Wye Branch

- D. Combination Elbow

- The building sewer conveys wastewater from the point of disposal to the building drain.

- A. False

- B. True

- The building drain is the lowest horizontal portion of the drainage system.

- A. True

- B. False

- The waste or soil stack is a horizontal pipe.

- A. True

- B. False

- The stack vent terminates through the roof to open air.

- A. False

- B. True

- The vent stack is a secondary vent that connects only to the sewer.

- A. True

- B. False

- A waste pipe carries wastewater with fecal matter.

- A. False

- B. True

- A soil pipe conveys discharge that may include fecal matter.

- A. False

- B. True

- The vent pipe ensures air circulation within the plumbing system.

- A. True

- B. False

- The stack is a vertical main of soil, waste, or vent piping.

- A. True

- B. False

- The branch is the main vertical line of the system.

- A. True

- B. False

- The building drain conveys wastewater to the house sewer.

- A. False

- B. True

- The building sewer extends from the house drain to the private sewage system.

- A. True

- B. False

- A building sewer may be less than 150 mm in diameter.

- A. False

- B. True

- Soil pipes should always be visible for maintenance.

- A. True

- B. False

- The location of fixtures should be considered during pre-planning stages.

- A. True

- B. False

- Soil pipe joints should be loose to allow flexibility.

- A. True

- B. False

- Metal hangers are used to anchor exposed soil pipes.

- A. True

- B. False

- Horizontal to horizontal direction changes should use 90° elbows.

- A. False

- B. True

- Fittings with more than one inlet at the same level are permitted.

- A. True

- B. False

- The combination wye and 1/8 bend are approved for direction changes.

- A. False

- B. True

- The ________ conveys sewage and wastewater from the building drain to the disposal point.

- A. Building Drain

- B. Building Sewer

- C. Soil Pipe

- D. Vent Stack

- The ________ is the lowest horizontal portion of the drainage system.

- A. Vent Stack

- B. Building Drain

- C. Waste Stack

- D. Sewer

- The vertical extension of the building drain is called the ________.

- A. Waste/Soil Stack

- B. Vent Pipe

- C. Branch

- D. Stack Vent

- The ________ begins where the stack ends and terminates to open air through the roof.

- A. Vent Stack

- B. Stack Vent

- C. Branch

- D. Building Sewer

- The main vent of the DWV system is known as the ________.

- A. Stack Vent

- B. Soil Stack

- C. Waste Stack

- D. Vent Stack

- The ________ pipe conveys wastewater without fecal matter.

- A. Soil

- B. Waste

- C. Vent

- D. Stack

- The ________ pipe conveys discharge from the water closet.

- A. Waste

- B. Soil

- C. Vent

- D. Branch

- The ________ pipe provides air circulation in the plumbing system.

- A. Drain

- B. Waste

- C. Vent

- D. Stack

- A vertical main that extends through stories and through the roof is called a ________.

- A. Stack

- B. Branch

- C. Drain

- D. Pipe

- Any part of the piping system other than a main or stack is a ________.

- A. Stack

- B. Vent

- C. Drain

- D. Branch

- The ________ receives discharge from the soil, waste, and other drainage pipes inside a building.

- A. Stack

- B. House/Building Drain

- C. Vent

- D. Sewer

- The ________ extends from the house drain to the public sewer.

- A. House/Building Sewer

- B. Stack

- C. Branch

- D. Vent

- A building sewer should not be less than ________ mm in diameter.

- A. 100

- B. 150

- C. 200

- D. 50

- The soil pipe should be properly ________ or embedded in walls.

- A. Concealed

- B. Exposed

- C. Painted

- D. Anchored

- The soil branch should be ________ and direct as practicable.

- A. Short

- B. Long

- C. Curved

- D. Extended

- Soil pipe joints must be tight and free from ________ leaks.

- A. Air

- B. Dust

- C. Rust

- D. Liquid or gas

- Exposed soil pipes should be anchored with ________ hangers.

- A. Wooden

- B. Metal

- C. Plastic

- D. Rope

- Horizontal to horizontal direction changes should use ________ branches.

- A. 90°

- B. 45°

- C. 30°

- D. 75°

- Fittings with more than one inlet at the same level are called ________.

- A. Sanitary Cross

- B. Wye

- C. Elbow

- D. Combination

- The combination wye and 1/8 bend are used for changes in ________.

- A. Pressure

- B. Flow rate

- C. Material

- D. Direction

- The size of waste or soil pipes depends primarily on:

A. The length of the pipe

B. The amount of waste it carries

C. The temperature of the waste

D. The type of material used

- A lavatory discharges how many liters per second?

A. 0.35 L/s

B. 0.47 L/s

C. 0.50 L/s

D. 0.75 L/s

- One lavatory fixture unit is equivalent to how many gallons per minute?

A. 3.5 GPM

B. 5.5 GPM

C. 7.5 GPM

D. 9.5 GPM

- Fixture unit rating of plumbing fixtures is based on:

A. The water pressure

B. The type of fixture

C. The size of the required trap

D. The pipe material

- 0.47 liters per second discharge is equivalent to how many liters per minute?

A. 25.3

B. 28.3

C. 30.3

D. 35.3

- Which of the following is equivalent to 1 cubic foot per minute?

A. 0.47 L/s

B. 28.3 L/min

C. 7.5 gallons per minute

D. All of the above

- The discharge capacity of 1 fixture unit is up to:

A. 0.35 L/s

B. 0.40 L/s

C. 0.47 L/s

D. 0.55 L/s

- The discharge capacity of 3 fixture units is between:

A. 0.5 to 0.95 L/s

B. 1 to 1.89 L/s

C. 1.95 to 3.15 L/s

D. Over 3.15 L/s

- The discharge capacity of 6 fixture units ranges from:

A. 0.95 to 1.50 L/s

B. 1.0 to 1.89 L/s

C. 1.95 to 3.15 L/s

D. 3.15 to 4.00 L/s

- Capacity over 3.15 L/s shall be determined by:

A. The engineer

B. The contractor

C. The Administrative authority

D. The property owner

- The size of waste pipes depends on the temperature of the waste.

A. True

B. False

- The amount of waste a pipe carries affects its size.

A. True

B. False

- A lavatory discharges 0.47 liters per second.

A. True

B. False

- One fixture unit equals 28.3 liters per minute.

A. True

B. False

- Fixture unit ratings are based on the pipe length.

A. True

B. False

- 7.5 gallons per minute is equivalent to one fixture unit.

A. True

B. False

- The discharge capacity of 2 fixture units is from 0.5 to 0.95 L/s.

A. True

B. False

- 6 fixture units have a discharge capacity between 1.95 and 3.15 L/s.

A. True

B. False

- Capacity below 0.47 L/s shall be determined by the Administrative authority.

A. True

B. False

- Capacity over 3.15 L/s requires Administrative authority approval.

A. True

B. False

- The size of waste pipes depends on the amount of ______ it carries.

A. Water

B. Waste

C. Pressure

D. Air

- A lavatory discharges ______ liters per second.

A. 0.47

B. 0.50

C. 0.35

D. 0.25

- One lavatory fixture unit is equivalent to ______ gallons per minute.

A. 5.5

B. 6.5

C. 7.5

D. 8.5

- The fixture unit rating is based on the size of the ______.

A. Drain

B. Trap

C. Pipe

D. Fixture

- 1 fixture unit has a discharge capacity up to ______ L/s.

A. 0.35

B. 0.40

C. 0.47

D. 0.55

- 3 fixture units have a discharge capacity between ______ L/s.

A. 1 to 1.89

B. 0.5 to 0.95

C. 1.95 to 3.15

D. 3.15 to 4.00

- Capacity over ______ L/s shall be determined by the Administrative authority.

A. 2.5

B. 3.15

C. 3.5

D. 4.0

- 2 fixture units have a discharge capacity between ______ L/s.

A. 0.35 to 0.55

B. 0.5 to 0.95

C. 1.0 to 1.89

D. 1.95 to 3.15

- The fixture unit rating depends on the size of the required ______.

A. Vent

B. Pipe

C. Trap

D. Valve